Design of an AI-Based Perception Module for Freight Truck Driver Assistance Systems

Sajid Ansari
Master of Computing, Artificial Intelligence
Curtin University

Abstract

This report studies the design of an AI-based perception module for freight truck driver-assistance systems. Freight trucks operate in complex and often risky environments, so reliable scene understanding is essential for safety. The proposed solution focuses on a monocular camera setup and covers three main tasks: object detection, semantic segmentation, and multi-task driving perception.

The report reviews key methods, including Faster R-CNN, DETR, Deformable DETR, RT-DETR, DeepLabV3+, SegFormer, ENet, BiSeNet, YOLOP, and HybridNets. Benchmarking is discussed using Cityscapes and BDD100K, while a proof-of-concept experiment is carried out using YOLOv8n on COCO128.

The experiment achieved a precision of 0.709, a recall of 0.632, an mAP@0.5 of 0.710, and an mAP@0.5:0.95 of 0.535, at 4.08 FPS on the CPU. A CodeCarbon run estimated emissions of 6.21 × 10⁻⁵ kg CO₂.

The findings show that multi-task perception is the most practical direction for freight-truck systems. It allows shared computation across tasks, but deployment still depends on hardware optimisation, real-time performance, and strong safety validation.

1. Introduction

Freight transport plays a key role in Australia’s economy. At the same time, driving conditions for heavy vehicles are difficult. Drivers deal with long distances, poor weather, night driving, and busy roads. These conditions make perception a challenging problem.

This report focuses on designing a perception module using a forward-facing monocular camera. The system aims to detect objects, understand the road scene, and support driver-assistance features such as warnings and alerts.

A practical system must satisfy four conditions:

- detect important objects like vehicles and pedestrians

- understand road structure, such as drivable areas

- run fast enough for real-time use

- remain reliable across changing conditions

These requirements create a trade-off. High-accuracy models are often slow, while fast models may miss details.

The objectives of this report are:

- analyse the perception problem

- review relevant methods

- compare datasets and models

- justify model choices

- present experimental results

- discuss ethics and deployment

2. Literature Review

2.1 Problem Taxonomy

The perception task can be divided into three parts.

Object detection
This task finds and classifies objects in the scene. Faster R-CNN provides strong accuracy using region proposals [3]. DETR later introduced a transformer-based approach [4]. Deformable DETR improved efficiency [5], while RT-DETR focuses on real-time performance [15].

Semantic segmentation
This task assigns a label to each pixel in the image. DeepLabV3+ is widely used for accurate segmentation [6]. SegFormer improves global understanding using transformers [7]. Lightweight models such as ENet and BiSeNet are designed for faster inference [8], [9], [10].

Multi-task perception
This combines detection and segmentation into a single model. YOLOP and HybridNets are examples that handle multiple tasks together [13], [14]. This approach is useful because it reduces duplicate computation.

2.2 Datasets for Benchmarking

Two datasets are commonly used.

- Cityscapes provides high-quality segmentation labels for urban scenes [1]

- BDD100K offers large-scale, diverse driving data with multiple tasks [2]

Cityscapes is strong for segmentation evaluation, while BDD100K is better for real-world diversity.

2.3 Model Comparison

Different models offer different trade-offs.

- Faster R-CNN is accurate but slow

- DETR simplifies the pipeline but requires more compute

- RT-DETR improves real-time performance

- DeepLabV3+ focuses on accuracy

- ENet and BiSeNet focus on speed

- YOLOP and HybridNets combine tasks efficiently

For freight trucks, a balanced solution is needed.

2.4 Key Trade-Offs

The literature shows clear patterns:

- higher accuracy usually increases latency

- faster models may lose detail

- multi-task models improve efficiency but increase complexity

- transformer models improve reasoning but require optimisation

3. Benchmarking Results

3.1 Metrics

The main metrics used are:

- Precision

- Recall

- mAP@0.5

- mAP@0.5:0.95

- FPS

These metrics reflect both accuracy and speed.

3.2 Experimental Results

A proof-of-concept experiment was run using YOLOv8n on COCO128.

Results:

- Precision: 0.709

- Recall: 0.632

- mAP@0.5: 0.710

- mAP@0.5:0.95: 0.535

- FPS: 4.08

- Evaluation time: 24.48 seconds

These results show that the model performs reasonably well on a small dataset

3.3 Interpretation

Three points stand out:

- the model learns meaningful patterns

- recall is lower than precision, meaning some objects are missed

- speed is too low for real-time deployment

At 4.08 FPS, the system cannot meet real-time CPU requirements.

3.4 Carbon Footprint

A CodeCarbon run estimated emissions of:

- 6.21 × 10⁻⁵ kg CO₂

This value is very small because the experiment used a lightweight dataset and a CPU.

Still, it shows how energy tracking can be included in AI workflows.

3.5 Best Model Choice

Multi-task models such as YOLOP or HybridNets are the most suitable.

They provide:

- shared feature extraction

- reduced computation

- better alignment with real driving needs

4. Methodology

4.1 System Design

The proposed system follows this pipeline:

- camera input

- preprocessing

- shared backbone

- detection + segmentation heads

- post-processing

- output to ADAS system

4.2 Model Strategy

YOLOv8n was used for testing because it is lightweight.

For real deployment:

- use multi-task architecture

- train on BDD100K and Cityscapes

- optimise for hardware

4.3 Data Strategy

Key practices include:

- image normalisation

- data augmentation

- handling class imbalance

- testing under different conditions

4.4 Deployment

CPU results show limitations.

To improve performance:

- use GPU or accelerator

- apply quantisation

- use TensorRT

- optimise model size

5. AI Ethics

The system must follow key principles:

- safety

- fairness

- privacy

- transparency

- accountability

For example:

- Missed detections can cause accidents

- datasets must be balanced

- camera data must be protected

6. Discussion

6.1 Sustainability

The system supports:

- safer roads

- improved transport systems

- reduced accident risk

6.2 Limitations

- The experiment used a small dataset

- CPU-only testing

- no full multi-task training

6.3 Future Work

Next steps:

- train on BDD100K

- build a multi-task model

- test on GPU

- evaluate edge cases

7. Conclusion

Freight-truck perception requires more than object detection. It needs a combined understanding of objects and road structure.

The experiment confirms that the pipeline works, but speed is not enough for real use.

The best direction is a multi-task, hardware-aware model that balances accuracy and speed.